Detailed Specification
1. HIKVISION IP Camera (2.0 MP, IR Night Vision, 1080p)

| Specification | Details |
| --- | --- |
| Model Reference | DS-2CD1xxxG0-I Series (Typical) |
| Image Sensor | 1/2.8" Progressive Scan CMOS |
| Effective Resolution | 2 Megapixels |
| Max. Resolution | 1920 × 1080 (Full HD 1080p) |
| Min. Illumination | Color: 0.01 Lux @ (F1.2, AGC ON); B/W: 0 Lux with IR |
| Shutter Speed | 1/3 s to 1/100,000 s |
| Lens | Fixed Focal: 2.8 mm / 4 mm / 6 mm options |
| Day & Night | IR Cut Filter (ICR) with Auto-Switch |
| Night Vision Range | Up to 30 meters (98 feet) |
| Video Compression | H.265+, H.265, H.264+, H.264 |
| Video Bit Rate | 32 Kbps to 8 Mbps |
| Audio | Built-in Microphone (select models) or Audio Input option |
| Network Protocols | TCP/IP, UDP, DHCP, DNS, DDNS, RTSP, NTP, etc. |
| Interface | 1 x RJ45 10M/100M Ethernet port |
| Power Supply | 12 V DC ± 25% or PoE (802.3af) |
| Power Consumption | Max 6W |
| Operating Conditions | -30°C to +60°C (-22°F to 140°F), Humidity 95% or less |
| Ingress Protection | IP67 (Weatherproof - Dust-tight and waterproof) |
| Weight | Approx. 400g - 600g (varies by form factor) |

### 2. HIKVISION 8 Channel Network Video Recorder (NVR)

| Specification | Details |
| --- | --- |
| Model Reference | DS-7608NI-K1 / 8P (Typical) |
| IP Camera Input | 8 Channel |
| Incoming/Outgoing Bandwidth | 80 Mbps / 80 Mbps |
| HDMI Output | 4K (3840x2160) @ 30Hz |
| VGA Output | 1920x1080 @ 60Hz |
| Recording Resolution | 12MP / 8MP / 5MP / 4MP / 1080p / 720p, etc. |
| Playback Capability | 8-channel synchronous playback at 1080p |
| Remote Access | HIK-Connect App (iOS/Android) & Web Browser support |
| Network Protocol | TCP/IP, DHCP, HIKVISION DDNS (No Static IP required), NTP, etc. |
| Network Interface | 1 RJ-45 10/100/1000 Mbps self-adaptive Ethernet port |
| SATA Interface | 1 SATA port (for the 1TB HDD) |
| HDD Capacity Support | Supports up to 10TB per HDD (depending on model) |
| USB Interface | Front: 1 x USB 2.0; Rear: 1 x USB 3.0 |
| Power Supply | 48 V DC or 12 V DC (depending on model) |
| Power Consumption | ≤ 10W (without HDD) |
| Dimensions (W x D x H) | 200mm x 200mm x 45mm (1U Mini Chassis) |
| Operating Temperature | -10°C to +55°C (14°F to 131°F) |

### 3. 1000 GB (1TB) Seagate Hard Disk

| Specification | Details |
| --- | --- |
| Model Series | Seagate SkyHawk (Recommended for Surveillance) / BarraCuda |
| Capacity | 1 TB (1000 GB) |
| Form Factor | 3.5 Inch |
| Interface | SATA 6Gb/s (SATA III - Backward Compatible) |
| Rotational Speed | 5900 RPM or 7200 RPM |
| Cache | 64 MB - 256 MB |
| Recording Technology | CMR (Conventional Magnetic Recording) |
| Workload Rate | Up to 180 TB/year (Optimized for 24/7 recording) |
| MTBF | Up to 1 Million hours |
| Special Features | Built-in RV sensors (SkyHawk models), Enhanced Write Caching for video streaming reliability. |
| Typical Usage | 24/7 DVR/NVR video recording and analytics. |

### 4. POE Switch 8 Port

| Specification | Details |
| --- | --- |
| Ports | 8 x RJ45 10/100/1000 Mbps (Gigabit Recommended) |
| PoE Ports | Ports 1-8 support PoE |
| PoE Standard | IEEE 802.3af (Up to 15.4W per port) / IEEE 802.3at (PoE+ Up to 30W) |
| PoE Power Budget | 60W - 120W (Total power for all cameras) |
| Network Standards | IEEE 802.3 (10BASE-T), 802.3u (100BASE-TX), 802.3ab (1000BASE-T) |
| Switching Capacity | 16 Gbps (Gigabit Model) |
| Packet Forwarding Rate | ~11.9 Mpps (Gigabit Model) |
| Mounting | Desktop / Rack-mountable (depending on chassis) |
| MAC Address Table | 2K - 8K entries |
| Jumbo Frame Support | Yes (up to 9K bytes for Gigabit models) |
| LED Indicators | Power, PoE Status, Link/Activity per port |
| Power Supply | Internal AC 100-240V, 50-60Hz |
| Operating Temperature | 0°C to 40°C (32°F to 104°F) |
| Operating Humidity | 10% to 90% non-condensing |