REQUEST FOR QUOTATION (RFQ)

Supply & Transportation of Bridge Materials – Waht-Garm Chashma, Lower Chitral

Aga Khan Rural Support Programme (AKRSP) invites sealed quotations from eligible, qualified and experienced suppliers/firms for the Supply and Transportation of Bridge Materials at Waht-Garm Chashma, Lower Chitral.

The procurement includes supply, packing, loading, transportation, unloading and delivery of structural steel, wire ropes, GI pipes and fittings, anchorage rods, plates, bolts, clamps and other bridge materials as specified in the RFQ/BOQ. All materials must be brand new, unused, free from defects and compliant with the specified technical requirements.

Key Requirements:

- Bidders must have at least two (2) similar completed supply and transportation contracts.

- Demonstrated cumulative completed supply experience of approximately PKR 5 million.

- Bid Security of 2.5% of the total quoted bid value is mandatory.

- Quoted rates must include all applicable taxes, duties, transportation, loading/unloading and other statutory/incidental costs.

- No advance payment shall be made. Payment will be made against specified delivery milestones, subject to satisfactory inspection and verification by AKRSP.

Submission Deadline: September 07, 2026, by COB

Sealed quotations shall be submitted by hand or reputable courier to:

Regional Office, AKRSP, Near Shahi Qilla, Chitral

The detailed RFQ, BOQ, technical specifications, terms and conditions can be accessed and downloaded from the link provided below.

Late, incomplete, unsigned, unstamped or non-responsive quotations may be rejected.

AKRSP reserves the right to accept or reject any or all quotations in accordance with its applicable procurement policies.

Zakir Hussain

Admin & Procurement Officer
AKRSP Chitral

Top of Form

Bottom of Form

AGA KHAN RURAL SUPPORT PROGRAM (AKRSP)

REQUEST FOR QUOTATION (RFQ)

RFQ No: AKRSP/0365/2026      Delivery Point: Charun, Upper Chitral

RFQ Date: August 27, 2026,       Submission Date: September 07, 2026

### Subject: Request for Quotation – Supply & Transportation of Bridge Materials at Waht-Garm Chashma, Lower Chitral

## Introduction

The Aga Khan Rural Support Programme (AKRSP) invites sealed quotations from eligible, qualified and experienced suppliers/firms for the Supply and Transportation of Bridge Materials at Waht-Garm Chashma, Lower Chitral.

The scope of this RFQ covers the supply, packing, loading, transportation, unloading and delivery of the materials listed in the BOQ to the designated project location. All materials shall be new, unused, free from defects and strictly in accordance with the

specifications stated in this RFQ.

## Quotation Submission Requirements

- Quotations shall be submitted in hard copy, on the bidder's original letterhead, duly signed and stamped.

- The RFQ number shall be clearly mentioned on the quotation and sealed envelope.

- Quotations shall be submitted to the AKRSP Regional Office near Shahi Qilla, Chitral, in the tender box at reception, by the submission deadline stated above.

- Late, incomplete, unsigned or unstamped quotations may be rejected.

- The quotation shall remain valid for 45 calendar days from the submission deadline.

- The bidder shall clearly indicate unit rates, applicable taxes and total prices.

- No conditional quotation shall be accepted unless specifically approved by AKRSP.

## Technical Evaluation – Pass/Fail

Only technically responsive bidders shall proceed to financial evaluation.

### Relevant Experience

The bidder shall provide documentary evidence of at least two (2) similar completed projects/contracts involving supply and transportation of structural steel, bridge materials, wire ropes, GI materials, steel sections or similar construction/bridge materials.

Evidence shall include, where applicable:

- Purchase Order/Contract Agreement;

- Completion Certificate or client acceptance certificate;

- Delivery/completion documentation; and

- Client name and contact details for verification.

Failure to provide satisfactory evidence of at least two similar completed projects may result in technical disqualification.

## Financial Capacity

## The bidder shall demonstrate sufficient financial and commercial capacity to undertake the required supply and transportation assignment by providing evidence of completed supply contracts with a total cumulative value of approximately PKR 5 million, either through a single completed supply contract or through multiple completed supply contracts. The bidder shall submit supporting documentary evidence, such as purchase orders, contract agreements, completion certificates, delivery challans, or client acceptance documents, clearly indicating the contract value, scope of supply, and completion status. AKRSP reserves the right to verify the authenticity of the submitted documents and references.

## Financial Evaluation and Award

Financial quotations of technically qualified bidders shall be evaluated based on:

- Total quoted price;

- Arithmetic accuracy;

- Completeness of the quotation;

- Compliance with specifications;

- Transportation costs;

- Applicable taxes and duties; and

- Overall price reasonableness.

Subject to AKRSP procurement procedures, the contract will ordinarily be awarded to the Lowest Evaluated Responsive Bidder (LERB).

AKRSP reserves the right to seek clarification regarding unusually high, low or inconsistent prices and to verify quoted prices against prevailing market rates.

## BOQ – Description of Works

Description of Works: Supply & Transportation of Bridge Materials at Waht-Garm Chashma, Lower Chitral

| S.No | Description of Goods | Specifications | Unit | Quantity | Unit rate Rs | Amount Rs |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Steel Wire Rope | 1" diameter, IWRC (6×19), EIPS, galvanized, new. Approximate weight: 0.839 kg/ft. Minimum breaking load: 93,000 lb (approximately 42.2 metric tonnes / 413 kN). | Ft | 1100 |  |  |
| 2 | Steel Girder (I-section) | Size: 8" × 4"; 20 Nos.; length of each girder: 20 ft; web thickness: 6 mm; flange thickness: 8 mm; approximate weight: 7 kg/ft; structural steel ASTM A36 or equivalent, ungalvanized. | kg | 2800 |  |  |
| 3 | Steel Girder (I-Section) | Size: 6" × 3"; 2 Nos.; length of each girder: 10 ft; web thickness: 6 mm; flange thickness: 8 mm; approximate weight: 5.25 kg/ft; structural steel ASTM A36 or equivalent, ungalvanized. | kg | 105 |  |  |
| 4 | C-Section Type -1 | Size: 5" × 2.5"; 92 Nos.; length of each section: 9 ft; web thickness: 6 mm; flange thickness: 7 mm; approximate weight: 3.5 kg/ft; structural steel ASTM A36 or equivalent, ungalvanized. | kg | 2898 |  |  |
| 5 | C-Section Type -2 | Size: 5" × 2.5"; 19 Nos.; length of each section: 13 ft; web thickness: 6 mm; flange thickness: 7 mm; approximate weight: 3.5 kg/ft; structural steel ASTM A36 or equivalent, ungalvanized. | kg | 865 |  |  |
| 6 | C-Section Type-3 | Size: 5" × 2.5"; 5 Nos., each 15 ft long; web thickness: 6 mm; flange thickness: 7 mm; approximate weight: 3.5 kg/ft; structural steel ASTM A36 or equivalent, ungalvanized. | kg | 263 |  |  |
| 7 | C-Section Type -4 | Size: 5" × 2.5"; 10 Nos., each 12 ft long; web thickness: 6 mm; flange thickness: 7 mm; approximate weight: 3.525 kg/ft; structural steel ASTM A36 or equivalent, ungalvanized. | kg | 420 |  |  |
| 8 | C-Section Type -5 | Size: 5" × 2.5"; 10 Nos., each 8 ft long; web thickness: 6 mm; flange thickness: 7 mm; approximate weight: 3.5 kg/ft; structural steel ASTM A36 or equivalent, ungalvanized. | kg | 280 |  |  |
| 9 | C-Section Type-6 | Size: 5" × 2.5"; 38 Nos.; length of each section: 3.5 ft; web thickness: 6 mm; flange thickness: 7 mm; approximate weight: 3.5 kg/ft; structural steel ASTM A36 or equivalent, ungalvanized. | kg | 466 |  |  |
| 10 | C-Section Type-7 | Size: 4" × 2"; 38 Nos.; length of each section: 2.5 ft; web thickness: 6 mm; flange thickness: 7 mm; approximate weight: 3 kg/ft; structural steel ASTM A36 or equivalent, ungalvanized. | kg | 285 |  |  |
| 11 | GI Pipes | 2" nominal diameter, Medium Class, hot-dip galvanized steel pipe, 22 Nos., each 20 ft long; wall thickness: 3.6 mm; approximate weight: 1.53 kg/ft | ft | 420 |  |  |
| 12 | GI Socket | 2" GI threaded equal socket/coupling, heavy-duty, hot dip galvanized, female threaded at both ends, suitable for 2" Medium Class GI pipe; BSP pipe thread | No’s | 18 |  |  |
| 13 | Anchorage Rod | 1½" diameter solid steel rods, 10 Nos., each fabricated from a 10 ft long ASTM A36 or equivalent structural steel rod and hot-dip galvanized after fabrication. One end of each rod shall be formed into a closed circular/O-shaped loop with a 3-inch internal diameter for wire-rope connection, while the other end shall be formed into a U-shape with 7-inch internal width and 4-inch internal depth for engagement with the 6" × 3" anchorage girder. The finished anchorage rods shall be of uniform length of approximately 8 ft | kg | 276 |  |  |
| 14 | Iron Plate | Size: 20" × 6", thickness: 8 mm; 30 Nos.; approximate weight: 4.85 kg per plate; mild/structural steel ASTM A36 or equivalent, ungalvanized. | kg | 146 |  |  |
| 15 | Nut bolt type-1 | ¾" diameter × 3" length, full threaded; high-strength structural bolt, Grade 8.8 (ISO 898-1) or ASTM A325 Type 1 or equivalent, ungalvanized; complete with matching nut and washer. | kg | 30 |  |  |
| 16 | Nut bolt type-2 | ½" diameter × 2" length, full threaded; high-strength structural bolt, Grade 8.8 (ISO 898-1) or ASTM A325 Type 1 or equivalent, ungalvanized; complete with matching nut and washer. | kg | 16 |  |  |
| 17 | Nut bolt type-3 | ¾" diameter × 2" length, full threaded; high-strength structural bolt, Grade 8.8 (ISO 898-1) or ASTM A325 Type 1 or equivalent, ungalvanized; complete with matching nut and washer. | kg | 12 |  |  |
| 18 | U-bolt type-4 | ½" diameter, 3" overall length, 2.5" clear spacing between legs, half-length threaded, drop-forged steel, hot-dip galvanized, complete with nuts and washers. | kg | 25 |  |  |
| 19 | Bulldog grips | Heavy-Duty Bulldog Grip, 28 mm, suitable for 26 mm galvanized steel wire rope, with forged steel U-bolt and saddle/base, complete with nuts and washers, hot-dip galvanized, BS EN 13411-5 / FF-C-450 Type 1 Class 1 / ASME B30.26 or equivalent, from a recognized manufacturer with size and identification markings. | No’s | 80 |  |  |
| 20 | GI Clamps | Fabricated from 3 mm thick galvanized iron sheet, 1" wide strip, with two ½" diameter holes. Internal circular diameter approximately 2.25" to provide clearance over the specified 2" GI pipe. Complete with suitable nuts, bolts and washers. | No’s | 114 |  |  |
|  | Total Cost |  |  |  |  |  |

Total Cost (PKR): __________________________

Note: The quantities mentioned are approximate and may vary depending on actual site requirements/conditions. Any variation shall be subject to AKRSP's approval.

## Bid Security

1. Bid Security Requirement

Each bidder shall submit a Bid Security equivalent to 2.5% of the total bid value along with its quotation. The Bid Security is intended to demonstrate the bidder’s commitment to its offer and to protect AKRSP against the risk of a bidder withdrawing or modifying its quotation during the bid validity period or failing to comply with the conditions applicable to award.

2. Acceptable Forms of Bid Security

The Bid Security shall be submitted in any one of the following forms:

- Bank Guarantee issued by a scheduled bank operating in Pakistan;

- Call Deposit Receipt (CDR);

- Pay Order;

- Demand Draft; or

- Banker's Cheque.

The Bid Security shall be made payable in favour of Aga Khan Rural Support Programme (AKRSP) and shall be submitted together with the quotation.

3. Validity of Bid Security

The Bid Security shall remain valid for at least twenty-eight (28) calendar days beyond the bid validity period.

4. Submission and Verification

The original Bid Security shall be submitted with the sealed quotation

AKRSP reserves the right to verify the authenticity and validity of the Bid Security with the issuing bank or financial institution. Any Bid Security that is found to be forged, invalid, expired, insufficient in amount, incorrectly issued, or otherwise not in compliance with the RFQ requirements may result in the bid being declared non-responsive.

5. Failure to Submit Bid Security

A bid submitted without the required Bid Security, or with Bid Security for an amount less than 2.5% of the total quoted bid value, may be considered non-responsive and rejected without further evaluation.

Similarly, a Bid Security that does not meet the prescribed form, validity, beneficiary or other mandatory requirements may result in rejection of the bid.

6. Return of Bid Security

The Bid Security of unsuccessful bidders shall be returned after completion of the evaluation process and award of the contract, subject to AKRSP's procurement procedures.

The Bid Security of the successful bidder shall be returned upon successful completion of all supply and transportation obligations and satisfactory inspection and verification by AKRSP.

7. Forfeiture of Bid Security

AKRSP may forfeit the Bid Security if the bidder:

- Withdraws or modifies its quotation during the bid validity period;

- Refuses to accept the award after being selected;

- Fails to sign the contract or Purchase Order within the period specified by AKRSP;

- Fails to provide any required contractual security or documentation after award;

- Provides false, forged, fraudulent or materially misleading information; or

- Engages in corrupt, collusive, fraudulent, coercive or obstructive practices during the procurement process.

8. No Interest

No interest, profit or other financial compensation shall be payable by AKRSP on the Bid Security, irrespective of the form in which it is submitted.

## Payment Terms

Payment shall be made through crossed cheque or bank transfer within fourteen (14) working days after achievement of the applicable milestone, satisfactory inspection/verification by AKRSP and submission of required documents.

First Payment – 60%:
Sixty percent (60%) of the total contract value shall be paid upon delivery of the materials to Chitral, subject to satisfactory inspection and verification by AKRSP, confirmation of the required quantities and specifications, and submission of the original invoice, delivery challan, and all relevant supporting documents.

Second Payment – 40%:
The remaining forty percent (40%) of the total contract value shall be paid upon successful delivery of all materials to Waht-Garm Chashma, Lower Chitral, subject to satisfactory verification by AKRSP, confirmation that all materials have been delivered in the required quantities and are free from transit damage, and submission of the final delivery documentation and invoice.

## No advance payment shall be made. All payments shall be made only after completion and verification of the respective delivery milestones by AKRSP. Applicable withholding taxes and other statutory deductions shall be deducted at source from the payable amount in accordance with the prevailing laws of Pakistan.

## Inspection and Quality Control

## All materials supplied under this RFQ shall be brand new, unused, and free from any defects. AKRSP reserves the right to inspect and verify the materials before dispatch, during transportation, and upon delivery. The supplier shall provide relevant manufacturer’s certificates, test reports, certificates of conformity, and other quality documents, where applicable. Any material found to be defective or not in compliance with the specified technical requirements may be rejected by AKRSP and shall be replaced by the supplier at its own cost, including all transportation and handling expenses. Inspection, verification, or payment by AKRSP shall not relieve the supplier of its responsibility for any latent defects, non-conformities, or quality issues identified subsequently.

### Taxes and Statutory Requirements

All quoted rates shall be inclusive of all applicable federal and provincial taxes, duties, levies, fees, transportation charges, loading and unloading costs, and other statutory or incidental charges payable under the prevailing laws of the Government of Pakistan and the relevant Provincial Government. The supplier shall be responsible for ensuring that its quoted prices fully account for all applicable tax and statutory obligations related to the supply and transportation of the materials.

The supplier shall provide valid NTN, sales tax registration details, KPRA registration where applicable, tax invoices, receipts, delivery challans, and any other documents required for compliance with applicable federal and provincial tax laws and regulations.

AKRSP shall deduct Withholding Tax (WHT), Sales Tax, KPRA tax, or any other applicable statutory deductions from payments, where required by the prevailing laws of Pakistan and the relevant provincial tax authorities, and shall deposit such deductions with the concerned government authority as required by law. Relevant tax deduction certificates shall be provided to the supplier where applicable.

The supplier shall be solely responsible for the accuracy and validity of all tax-related information and documents submitted to AKRSP. Any additional tax liability, penalty, interest, fine, or other financial obligation arising due to incorrect, incomplete, invalid, or misleading information provided by the supplier shall remain the supplier's responsibility and shall not be payable by AKRSP.

## Transportation, Safety and Liability

The supplier shall use suitable and roadworthy vehicles and shall ensure proper loading, securing, transportation and unloading of materials.

The supplier shall comply with applicable traffic, transportation, health and safety requirements. Any loss, damage, accident or injury arising from the supplier's transportation or handling activities shall remain the supplier's responsibility.

The supplier shall maintain appropriate insurance for its personnel, vehicles, goods and third-party liabilities, as applicable.

### Anti-Fraud, Anti-Collusion and Conflict of Interest

AKRSP maintains zero tolerance for fraud, corruption, collusion, bid-rigging, and other prohibited practices. The bidder shall provide complete, accurate, and genuine information and shall conduct the bidding process in a fair and transparent manner. AKRSP reserves the right to reject a quotation, disqualify a bidder, or terminate the contract if the bidder submits false, forged, misleading, or fraudulent information; engages in collusion or bid-rigging; attempts to improperly influence the evaluation or award process; has any undisclosed actual or potential conflict of interest; or engages in corrupt, fraudulent, coercive, or obstructive practices. Any such action may also result in forfeiture of bid security and/or other remedies available to AKRSP under the applicable procurement rules and contract.

### Disqualification

A quotation may be rejected or declared non-responsive if it is submitted late, incomplete, unsigned, or unstamped, or if it does not include the required Bid Security. A bidder may also be disqualified if it fails to meet the mandatory technical specifications or provide the required evidence of relevant experience, submits unauthorized conditions or deviations, provides false, forged, or misleading information, does not comply with the required quotation validity period, or fails to meet any other mandatory requirement of this RFQ.

### AKRSP's Rights

AKRSP reserves the right, in accordance with its applicable procurement policies, to accept or reject any or all quotations without assigning a reason, cancel, suspend, or re-advertise the procurement process, seek clarification from bidders, and verify any information, documents, references, or claims submitted by the bidders. AKRSP also reserves the right to reject any materials that do not comply with the required specifications and to award the contract to the bidder whose quotation is determined to be substantially responsive and provides the best value for money, taking into consideration price, technical compliance, quality, experience, and capacity to deliver.

Contractor's Accommodation, Travel, and Subsistence

The contractor shall be solely responsible for arranging and bearing all costs related to accommodation, meals, travel, local transportation, fuel, communication, and other logistical requirements for its personnel throughout the execution of the contract. AKRSP shall not provide or reimburse any such expenses, including accommodation, food, travel, transportation, lodging, or other incidental costs, unless expressly agreed and stated in writing in the contract.

## Force Majeure and Governing Law

Neither party shall be liable for failure or delay caused by events beyond its reasonable control, including natural disasters, war, civil unrest, government restrictions or other force majeure events.

The contract shall be governed by the laws of the Islamic Republic of Pakistan. Disputes shall first be resolved amicably through consultation; failing such resolution, the matter may be referred to the competent courts having jurisdiction in Chitral.

### Submission Deadline

Sealed quotations must be delivered by hand or through a reputable courier service to the Regional Office, AKRSP, Near Shahi Qilla, Chitral, no later than Close of Business (COB) on September 07, 2026. Quotations received after the specified deadline shall not be opened and shall be returned unopened where practicable.

We appreciate your interest in partnering with AKRSP in this community-driven initiative.

Admin & Procurement Officer
AKRSP Chitral

### Bidder's Declaration and Vendor Receipt & Acknowledgement

By submitting a quotation, the bidder confirms that it has carefully read, examined, and understood the complete RFQ document, including the scope of supply and transportation, BOQ, technical specifications, delivery requirements, payment terms, bid security requirements, evaluation criteria, taxes, and all other terms and conditions. The bidder further confirms that the quoted rates are based on a full understanding of the requirements and that the bidder has satisfied itself regarding the nature and extent of the required supply and transportation. The bidder acknowledges that all information and documents submitted with the quotation are true, accurate, complete, and valid, and agrees to comply with all applicable requirements of this RFQ and the prevailing laws and regulations of Pakistan. By signing below, the bidder confirms its acceptance of the terms and conditions of the RFQ and undertakes to fulfill its contractual obligations if awarded the contract by AKRSP.

### Vendor Receipt & Acknowledgement

I/We hereby acknowledge receipt of the RFQ document and confirm that I/we have read and understood its contents and agree to comply with the requirements, terms, and conditions stated therein.

Name of Authorized Representative: ______________________________

Designation: _________________________________________________

Company/Firm Name: ___________________________________________

CNIC/NTN (if applicable): ______________________________________

Contact No.: _________________________________________________

Email: _______________________________________________________

Signature & Official Stamp: ____________________________________

Date: _______________________________________________________

Picture of U-Bolt with Washers

Bulldog Grip with forged U bolt and Saddle

steel U bolt & Saddle

Picture of Anchorage Rod

Picture of G.I Clamp